RFID Based Attendance System

Abstract

This project uses an Arduino Uno, MFRC522 RFID reader, LCD display, LEDs, buzzer, and RFID cards to automatically record attendance. When a valid RFID card is scanned, the system displays the user ID, gives a success indication, and marks attendance.

Objectives

• Automate attendance
• Reduce manual errors
• Fast identification using RFID
• Display attendance status on LCD

Components

Arduino Uno
MFRC522 RFID Reader
RFID Cards/Tags
16x2 I2C LCD
Buzzer
Red LED
Green LED
Jumper Wires
Breadboard
USB Cable

Block Diagram

RFID Card -> MFRC522 -> Arduino Uno -> LCD Display
                               -> LEDs
                               -> Buzzer

Working Principle

1. User places RFID card near the reader.
2. MFRC522 reads the UID.
3. Arduino compares the UID with stored IDs.
4. If valid, attendance is marked and green LED/buzzer operate.
5. If invalid, red LED indicates access denied.

Pin Connections

MFRC522: SDA-D10, SCK-D13, MOSI-D11, MISO-D12, RST-D9, 3.3V, GND.
LCD(I2C): SDA-A4, SCL-A5, VCC-5V, GND.
Green LED-D7.
Red LED-D6.
Buzzer-D5.

Advantages

Fast, accurate, paperless, easy to use, low cost.

Disadvantages

Limited reading range, card loss/misuse possible.

Applications

Schools, colleges, offices, industries, libraries.

Future Scope

Cloud database, Wi-Fi/IoT integration, fingerprint + RFID, mobile app monitoring.

Conclusion

The RFID attendance system provides a reliable and efficient method for recording attendance while reducing manual effort.