#include <SPI.h>

#include <MFRC522.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#define SS_PIN 10

#define RST_PIN 9

#define GREEN_LED 7

#define RED_LED 6

#define BUZZER 5

MFRC522 mfrc522(SS_PIN, RST_PIN);

LiquidCrystal_I2C lcd(0x27, 16, 2);   // Change to 0x3F if your LCD uses that address

// Replace this with your RFID card UID

String validUID = "12345678";

void setup() {

Serial.begin(9600);

SPI.begin();

mfrc522.PCD_Init();

lcd.init();

lcd.backlight();

pinMode(GREEN_LED, OUTPUT);

pinMode(RED_LED, OUTPUT);

pinMode(BUZZER, OUTPUT);

digitalWrite(GREEN_LED, LOW);

digitalWrite(RED_LED, LOW);

digitalWrite(BUZZER, LOW);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("RFID Attendance");

lcd.setCursor(0, 1);

lcd.print("Scan Your Card");

Serial.println("Scan RFID Card...");

}

void loop() {

if (!mfrc522.PICC_IsNewCardPresent())

return;

if (!mfrc522.PICC_ReadCardSerial())

return;

String uid = "";

for (byte i = 0; i < mfrc522.uid.size; i++) {

if (mfrc522.uid.uidByte[i] < 0x10)

uid += "0";

uid += String(mfrc522.uid.uidByte[i], HEX);

}

uid.toUpperCase();

Serial.print("UID : ");

Serial.println(uid);

lcd.clear();

if (uid == validUID) {

lcd.setCursor(0, 0);

lcd.print("ACCESS GRANTED");

lcd.setCursor(0, 1);

lcd.print("ATTENDANCE OK");

digitalWrite(GREEN_LED, HIGH);

digitalWrite(BUZZER, HIGH);

delay(300);

digitalWrite(BUZZER, LOW);

delay(2000);

digitalWrite(GREEN_LED, LOW);

} else {

lcd.setCursor(0, 0);

lcd.print("ACCESS DENIED");

lcd.setCursor(0, 1);

lcd.print("INVALID CARD");

digitalWrite(RED_LED, HIGH);

digitalWrite(BUZZER, HIGH);

delay(1000);

digitalWrite(BUZZER, LOW);

digitalWrite(RED_LED, LOW);

}

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Scan Your Card");

mfrc522.PICC_HaltA();

mfrc522.PCD_StopCrypto1();

}